| BOQ for Construction of 02 Block Institutional Latrine (04 stance) in Um Kowaika and Salima health centers in Tendalty locality: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Item | Description | Unit | Quantity | Price SDG | Total SDG |
| 1 | Pit Excavation |  |  |  |  |
| 1.1 | Excavation of latrine pit 6x3x3.6 Meter deep for each latrine as per the drawing (well shaping for the edges) | M³ | 129.6 |  |  |
| Sub Total Excavation |  |  |  |  |  |
| 2 | Bottom Slab and pit lining |  |  |  |  |
| 2.1 | Supply and cast 10 Cm plane concrete at the bottom 1:3:6 Mix as per design, including the curing for at least 03 days, 6*3 M for each latrine | M3 | 3.6 |  |  |
| 2.2 | Supply and cast 15 Cm reinforced Concrete with 12 mm rebar, spacing 15 Cm C/C (1:2:4) - Bottom Slab (6*3 Meter) for each latrine, including curing with water for 03 days at least 03 times per day. | M3 | 5.4 |  |  |
| 2.3 | Supply and build the pit lining (1.5 brick thick wall (30cm) using Red Burnt Bricks in 1:6 cement sand mortar mix up to 45 Cm above natural ground level with construct one partition in the middle of pit with keep hole 50*50 cm in the middle of partition for both two latrines. The dimension for each latrine is 6*3*4 m hight | M² | 168 |  |  |
| 2.4 | Supply, backfill and compact selected red soil around all the pit excavation between the wall and the pit for both two latrines | M³ | 4 |  |  |
| Sub Total Bottom Slab and pit lining |  |  |  |  |  |
| 3 | Top Slab (6.4*3.4*0.15) |  |  |  |  |
| 3.1 | Supply, fill, and compact red soil around the ramp, steps, and the extended slab Meter, as per drawing above the ground level. For both two latrines | M³ | 14 |  |  |
| 3.2 | Supply and Cast -Reinforcement concrete slab (1:2:4 concrete mix), 15 Cm thick, 6.4*3.4 Meter, covering latrine pit using 12mm diameter Rebar, 15cm spacing between the bars, including smooth finishing, including provision for 02 manhole with cover 0.5 *0.5 Meter, including curing for at least 03 days 03 times per day with very smooth finishing. For both two latrines | M3 | 6.5 |  |  |
| 3.3 | Supply and place -Plain concrete 5 Cm thickness (1:3:6) for Access Ramp and steps for both two latrines | M³ | 1.2 |  |  |
| 3.4 | Supply and install a handrail 75 Cm high as 03 lines for the ramp and one room for each latrine as per drawing, of painted metallic circular bar 1 inch, 1.5 mm thickness, 6 meters long. | Pcs | 8 |  |  |
| Sub Total top slab |  |  |  |  |  |
| 4 | Superstructure (6*1.4*3 meter) |  |  |  |  |
| 4.1 | Walling: Supply and build one brick-thick wall plus barabit 0.4 Meter above the roof, using Red Burnt Bricks in 1:6 cement-sand mortar mix, including the provision for ventilation/lighting (04 barabikh 0.3*0.4 M) bricks for 4 windows and curing for at least 03 days, 03 times per day. For both two latrines | M² | 100 |  |  |
| 4.2 | Supply and fix on the walls a Solar lamp, 200-Watt power all-in-one with a sensor for sunlight, including a remote control (4 inside and 02 outside) for both two latrines | Pcs | 12 |  |  |
| 4.3 | Wall plastering: internal and external including curing for at least 03 days. | M² | 200 |  |  |
| 4.4 | Roofing: Supply and install 4*8 cm painted rectangular bar, 6 meters long - 0.9 mm thick on the top of the wall to be ready to receive the zinc sheet (2 lines will be installed with the long span, 6 meters, and 02 barrels with the short span, 1.5 meters long). For both two latrines | Pcs | 6 |  |  |
| 4.5 | Roofing: Supply and install 7ft 0.35mm Sudani type Zink sheet as a roof for each latrine | Pcs | 18 |  |  |
| 4.6 | Supply and install -PVC vent pipe- 75mm diameter, 4mm thick - 3 meters long + Caps (two pipes for each latrine) | Pcs | 4 |  |  |
| 4.7 | Supply and install Squatting Pans 3 for each latrine | Pcs | 6 |  |  |
| 4.8 | Supply and install direct sitting pan (without water tank). One for each latrine | Pcs | 2 |  |  |
| 4.9 | Curtain Wall: Supply and build one brick thick wall using Red Burnt Bricks in 1:6 cement sand mortar mix curing for at least 03 days, 03 time per day (6*2 Meters for each latrine) | M² | 24 |  |  |
| 4.10 | Curtain Wall: Supply and plaster the wall external and internal including curing for 03 days. For both two latrines | M² | 48 |  |  |
| 4.11 | Supply and cast the area between the curtain wall and the latrine with 10 Cm concrete (6*1.4 M), 1:3:6 mix. For both two latrines | M² | 18 |  |  |
| 4.12 | Supply and paint a pomastic paint silk type, 03 coats (Almohandis or Equivelant) for both two latrines | M² | 248 |  |  |
| Sub Total Superstructure |  |  |  |  |  |
| 5 | Doors |  |  |  |  |
| 5.1 | Fabricated metallic- painted framed door; Size: 0.8*2 Meters high overall, complete with all hinges, lock, handle, and other accessories, well painted, the frame made from rectangular bar 3*6 cm, 0.9 mm thickness, iron sheet 0.6 mm thickness. | Pcs | 6 |  |  |
| 5.2 | Ditto but 1*2 Meter for Disabled person’s room. | Pcs | 2 |  |  |
| Sub Total Doors |  |  |  |  |  |
| 6 | Visibility/ Signpost |  |  |  |  |
| 6.1 | Manufacture and erect metal signpost made from steel blade sheet 3 mm thick and 120c X80 cm welded and fixed to 2.5-meter length rectangular steel pipe 3cm X 6cm, 2 meter over ground level and 50 cm underground level and erected on concrete base 0.5X0.5X60cm all other details will be provided by the project | Pcs | 2 |  |  |
| Subtotal Visibility/ Signpost |  |  |  |  |  |
| Total - 02 blocks of Latrine SDG |  |  |  |  |  |