Provide comprehensive WASH, Shelter/NFI, and Protection interventions to mitigate morbidity and mortality among children and their families in response to the Sudan crisis

Bid documents

NORTH KORDOFAN STATE

SHIKAN LOCALITY

Construction of Three semi-permanent Latrines in Al-mena Al-pari “Land Port” IDPs camp – Shikan locality

Civil Works

Metal Works

January 2026

المواصفات الفنية

# Specifications & Performance Requirements

General Provisions

Scope and Applications

These Specifications apply to the construction of three semi-permanent latrines four chamber pits, for each. The Works under this Contract for latrines comprises for each latrine: Excavation of the pits, construction of beam foundation, construction of the bids using red brick and sand cement mortar, construction of reinforced concrete slabs fixed on the foundation and made lining with 2 barriers at the middle of the pit roofing the bids using reinforcement concrete, construction of 4 rooms with 4 pits using zinc sheet, furnished with single 4 Doors and 4 small ventilation windows, to the details as specified below.

The Latrines should be completed with the below basic specifications:

The Latrines:

1. The offer should be supported by technical data printout

2. Latrine should be located at the west or east direction of the site with 4 pits.

3. Latrine should be located 30m maximum from the nearest water source.

4. Latrine should be located 30m maximum from the nearest Kitchen.

5. The latrine should satisfy the user privacy

6. The latrine should be constructed based on MOH & UNICEF and WASH Sector in NKS standards and technical specifications

7. The excavated soil should be compacted around the latrine for more protection

The Pit:

8. The Pit must be lined completely from the inside so that the hole and the room do not collapse

9. Pit dimensions was 4 length*3width*4Depth

The Foundation:

10. Concrete slabs grounded on it. The Foundation is used to prevent rodents and surface water from entering the pit, and it must be 15 cm above the surface adjacent to it

The Concrete Slabs/Floor:

11. Used to carry the user and to cover the hole, construct tightly with the foundation so that the edges of the slabs are identical to the outer edge of the foundation, construct of the concrete and use to provide with two openings:

a. Slot seat is the key shape length is 35 --- 40 cm and 18 cm in diameter, must be provided with foot positions and be 20 degrees slope
b.  Circular or squire ventilation opening according to the shape of the ventilation pipe

Pit elevation from adjacent land surface:
13. The height of the constructed pit should be 50 cm from all sides above to the height of the land surface (NGL) adjacent to it

The Latrine’s room:

14. The size of the room should be proportional to the dimensions of the floor so as not to be large enough to encourage the user to defecate at any part of the floor

15. The entrance length of the room shall be at least 180 cm

16. Ventilation of the room through a hole in the upper walls of the room

17. Room design should provide sufficient natural lighting

18. The Zinc roof shall provide full protection for the room and shall have a large umbrella to protect the top

19. Make opening for the vents and holes for discharging the craps

Ventilation pipe: -
20. It should be long enough that the latrine’s roof does not affect the flow of wind at the top of it and that it should be above the ceiling by about 50 cm
21. The ventilation pipe must be placed outside the Latrine room facing the sun
22. A fly trap should be installed on the top of the pipe (a mesh to prevent the passage of flies and mosquitoes) and that the size of the mesh openings should not be more than 1.5 ---- 1.2 mm
23. To avoid bad ventilation, the mesh should be made of corrosion resistant material and can withstand heat, rain, sunlight and gases.

Construction Specifications:
24. The Beam should be done down the latrine’s hole
25. Construction of latrine’s pit using brick and cement mortar and making of one barrier in the center of the hole and leave an area of 50 * 50 cm between them

26. Concrete mixtures should be 4:2:1
27. The thickness of the tile should be 15 cm to be fixed on the reinforced concrete slab (Floor) and half of the room walls

28. The area of the latrine’s room should be 100 length, *110 cm width and 2.5 meters height
29. Door dimensions should be 60 width* 180 cm height
30. Dimensions of the window should be 40 width* 40 cm height
31. Openings for the Vents and openings for waste/crap discharging should be made on the reinforced concrete slabs.

32. The latrines room and roof should be made of heavy Zinc sheet 0.35mm.

CONSTRUCTION OF THREE SEMI PERMANENT LATRINES IN ALOBAYED LAND PORT IDPs CAMP IN SHIKAN LOCALITY- NORTH KORDOFAN STATE

- Bill of quantities for construction of the three semi-permanent latrines

- Mobilization

Bill of Quantity 1-1 for moblization

| # | DESCRIPTION | QUANTITY | unit | UNIT PRICE SDG | TOTAL PRICE SDG |
| --- | --- | --- | --- | --- | --- |
| 1 | Mobilization to the site: Mobilization shall consist of preparatory work including, but not limited to, those necessary to the movement of personnel, equipment, supplies and incidentals to the project site in Al-mean Al-pari “Land Port” IDPs camp. | 1 | JOB |  |  |
| Total of Bill No. 1-1 |  |

- Civil works

| # | Items descriptions | Quantity | Unit | Unit Price SDG | Total Price SDG |
| --- | --- | --- | --- | --- | --- |
| 1 | Excavation of the three pits with dimensions of 4*3*4 m for each latrine, as per specifications stated above or the instructions of the WASH coordinator to reach the stated capacity | 144 | M3 |  |  |
| 2 | Fill & compact the area close to the latrines by the excavated soil after construction of the foundation, lining and floor (Concrete slab) as per specifications stated above or the instructions of the WASH Coordinator | 72 | M3 |  |  |
| 3 | Supply and cast of plain concrete for the latrine pits with dimensions 4m*3m*10 cm thickness for each latrine to carry the walls of the lining using plain concrete (1:3:6) as per above specifications or the instructions of the WASH Coordinator | 36 | M2 |  |  |
| 4 | Provide materials & build a rectangular bricks walls 20 cm thickness and in 1:6 cement sand mortar for Pit’s lining with one partition including holes in the middle of partitions with dimensions 50*50 cm, the walls should be elevated 50cm above the land surface adjacent to it outside the Pit, the wall should be plastered using 1:8 cement sand mortar | 3 | Job |  |  |
| 5 | Supply Construction of reinforced concrete slab (floor) with dimensions of 4*3*0.15 m with eight holes four as slot seat, two for dislodging (40*40cm) and two as ventilation using reinforcement concrete (1:2:4) with 12 mm bars distributed 20 cm in two sides as per the above specifications or the instructions of the WASH Coordinator | 5.4 | M3 |  |  |
| 6 | Plumbing works: Supply and install high quality pipe 4” 8 bar with all necessary fittings and covered with fly trap as per above specifications for the ventilation pipe. The ventilation pipes should be fixed with a length of 50 cm above the latrine roof, two ventilation for each latrine. | 6 | PC |  |  |
| 7 | Supply and fix on the walls a Solar lamp, 1500Watt power all-in-one with a sensor for sunlight, including a remote control (4 outside) for both three latrines | 12 | PC |  |  |
| Total of Bill for civil work |  |  |

3. Metal Works

Bill No. 3-1 Latrine rooms, doors and windows construction

| # | Items descriptions | Quantity |  | Unit |  | Unit Price SDG | Total Price SDG |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Building of the room with area of 100 cm length*110 cm width and 2.5m height using Zinc sheet 0.35 ml and rectangular pipes 4*8 thickness 1”. Supply and cast 12 cement seats on the 12 pits for the three latrines. The three latrines should have roofed by Zink sheet 0.35mm and heavy metal pipes 4*8. For each latrine one room should adding disability facilities such as slope floor (ramp) and hand ring from outside the latrine to facilitate disabled people to use the latrine. Above specifications according to the instructions of the WASH Coordinator. |  | 3 |  | latrine |  |  |
| 2 | Latrine Doors: Supply and installation of Single Doors shall be manufactured using appropriate heavy Zinc sheet 0.35mm and angel metal lockable welded to squire heavy iron pipes frame 3*6 cm. The dimensions of the Door was 180 cm height*60 cm width with painting |  | 12 |  | PC |  |  |
|  | Construction small Windows: Single small window should be manufactured using appropriate heavy expended metal and lockable welded to heavy metal angle frame. the dimensions of the Window was 40 cm height*40 cm width |  | 12 |  | PC |  |  |
| 3 | Manufacture and erect metal signpost made from steel blade sheet 3 mm thick and 120c X80 cm welded and fixe to2.5 meter length rectangular steel pipe 3cm X 6cm, 2 meter over ground level and 50 cm underground level and erected on concrete base 0.5X0.5X60cm all other details will be provided by the project |  | 3 |  | PC |  |  |
| 4 | Painting of organization and donor logos, along with hygiene promotion guidelines, on each latrine. |  | 3 |  | JOB |  |  |
| Total of Bill 3 |  |  |

###### COST SUMMARY FOR THREE SIMI PERMANENT LATRINE CONSTRUCTION PROJECT (Summary of Tender Price)

| Item | Description | Cost (Sudanese Pounds)/SDG |
| --- | --- | --- |
| ### 1 | Mobilizations |  |
| ### 2 | Civil Works |  |
| ### 3 | Metal Works |  |
| ### 4 | Total amount of work |  |
| ### 5 | Vat amount |  |
| Total amount of tender |  |  |