Provide comprehensive Emergency Shelter/NFI, and Protection interventions to mitigate morbidity and mortality among children and their families in response to the Sudan crisis - NKS, Shiekan locality

Bid documents

North Kordofan State

Shikan Locality

Supply of local materials for construction of 750 emergency shelters in Almainaa Albarry camp in Alobayed

NFIs

# Specifications & Performance Requirements

General Provisions

1-Scope and Applications

These Specifications apply to supply of local materials for construction of 750 emergency shelters in Alminaa Albarri IDPs camp in Alobayed as detail as specified below.

2- Requirements of Specifications, Standards and Brand Names

The supplier shall fulfil all requirements and obligations under all Clauses of the Specifications. Neither the following Clauses of this Specification, any descriptions therein nor the quantities shall limit the obligations of the Contractor under the Conditions of Contract. Where items are not included in the Bill of Quantities for any such requirements or obligations, the cost of such requirements or obligations shall be deemed to be spread over all the items of the Bill of Quantities. All Sudanese, or other Standards mentioned herein shall be deemed to form part of this Specification. All references to such standards shall be to the latest edition or revision thereof unless otherwise stated. Where a specific Sudanese or other Standard is referred to in this Specification, another Standard will be acceptable, provided that it ensures a quality of material and workmanship equal to or better than the Standard referred to. If the Contractor intends to use such alternative Standard, he shall notify the Sector coordinator thereof. Submitting with his notice two (2) copies (in English or Arabic) of the proposed Standard and shall not order any material or perform any work unless and until he has obtained the Sector Coordinator’s approval of such Standard. Brand names, where used in the Specification or on the drawings, are only intended to define a standard of quality and performance and the Contractor may use alternative products of at least equal quality and capacity. When the Contractor offers alternatives, the supplier shall submit to the Sector Coordinator for approval a statement detailing the alternatives, and shall include full technical descriptions, drawings and specifications, and shall provide such full information as is required to enable the Contractor to demonstrate to the Sector Coordinator that the alternative is equivalent to the item specified. The Contractor when called for shall produce any further information that the Sector Coordinator may require.

3. Approval of Suppliers, Services, Materials and Goods

All materials to be provided shall be new, unused, of most recent manufacture and incorporate all recent improvements in the design and material unless provided otherwise in the BOQ. Physical check to all material and tools by the organization engineers and the specification certificate must receive.

BOQ for Supply of local materials for construction of   750 emergency shelters in Alminaa Albarri IDPs camp

| # | Item البيان | Unit الوحدة | # of unit عدد الوحدات | Unit /shelter العدد لكل خيمة | Unit cost / SDG سعر الوحدة | Total cost / SDG سعر الجملة |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2.5m side Hardwood with 8 cm diameter of either of Ban, Sahab or Abanus tree, fresh local harvest, with unstripped bark, free from decay, insect attack, rot pockets and straight with no clear band. عود جنب (مداد) بطول 2.5 متر وبقطر 8 سم من شجرة البان أو الصهب أو الابانوس، مع لحاء غير مقشر، خالٍ من التسوس والهجوم الحشري والتعفن ومستقيم بدون تعرجات واضحة. | pcs | 6,000 | 8 |  |  |
| 2 | 3 meter length Hardwood poles with 8 cm diameter of either of Ban, Sahab or Abanus tree, fresh local harvest, with unstripped bark, free from decay, insect attack, rot pockets and straight with no clear band. عود (شعبة) بطول 3 أمتار وقطر 8 سم من أشجار البان أو الصهب أو الأبانوس، حاصلة على حصاد محلي طازج، بقشرة غير مقشرة، خالية من التعفن والتسوس وهجمات الحشرات والجيوب الفاسدة ومستقيمة بدون تعرجات واضح. | pcs | 2,250 | 3 |  |  |
| 3 | 5-6 meter length Hardwood beam with 2.5 inch diameter of either of Ban, Sahab or Abanus tree, fresh local harvest, with unstripped bark, free from decay, insect attack, rot pockets and straight with no clear band. عارضة صلبة بطول 65- أمتار وقطر 2.5 بوصة من شجرة البان أو الصهب أو الأبانوس، خالية من التعفن أو هجوم الحشرات أو جيوب التعفن، مستقيمة بدون تعرجات واضح. | pcs | 3,750 | 5 |  |  |
| 4 | 4 meter Bamboo stick with 1.5 inch diameter for the roof عصا قنا بطول 4 أمتار وقطر 1.5 بوصة للسقف | pcs | 12,000 | 16 |  |  |
| 5 | 3.2x2m grass mat with good quality grass for covering the shelter structure شرقانية عشب بمقاس2*3.2متر مع عشب عالي الجودة | pcs | 8,250 | 11 |  |  |
| 6 | Bending wire سلك رباط | Kg | 1,500 | 2 |  |  |
| 7 | 1.8m hieghtx1m width of bamboo Waven door باب من القنا بارتفاع 1.8 متر وعرض 1 متر | pcs | 750 | 1 |  |  |
| 8 | thick Plastic Sheet 4*8 | pcs | 1,500 | 2 |  |  |
| 9 | Transportation of the local materials to Alminaa Albarri camp and put each shelter materials in specific place for construction ترحيل مواد البناء الي الموقع بمعسكر الميناء البري مع وضع مواد تشييد أي خيمة بموقع الخيمة المحددة وفق الكروكي المرفق | Job | 750 | 1 |  |  |
| Total Amount of tender |  |  |  |  |  |  |

المدة الزمنية للتسليم: